Directions

- Carefully read the overhead projection for any additional directions specific to this quiz/exam.

- In the header, replace the highlighted text with your name and remove the highlight.

- Make your final answers clear, ideally with circles or in green. Correct answers made clear “after the fact” will not be graded.

- Any questions with an answer but no justifying work will be graded as incorrect.

X

(Exceptions are made for those with accommodations requests.)
>>>>>>  ALL TEXT OR OTHER FORMS OF ANSWERS WILL BE IGNORED!!!!!!!! <<<<<<<
>>>>>> THIS INCLUDES POWERPOINT AND OTHER VECTORIZED DRAWINGS !!!!! <<<<

- Violating any of these rules or other malicious behavior will result in a 0 grade and disciplinary action report to the provost’s office.

- Partial credit is awarded for flawed work or answering the question outside of given constraints: do not leave an answer blank if you can answer a similar question.

- Any indication of copied work, especially identical incorrect answers or identically formatted answers will 1) be marked as incorrect and 2) be referred to the Provost’s Office for academic misconduct.

- Submit your final answers as a PDF document by the deadline.

|  | Expression | Dual |
| --- | --- | --- |
| P2 | a+0=a | a⋅1=a |
| P3 | a+b=b+a | ab=ba |
| P4 | a+b+c=a+b+c | abc=abc |
| P5 | a+bc=(a+b)(a+c) | ab+c=ab+ac |
| P6 | a+a=1 | a⋅a=0 |
| T1 | a+a=a | a⋅a=a |
| T2 | a+1=1 | a⋅0=0 |
| T3 | a=a |  |
| T4 | a+ab=a | aa+b=a |
| T5 | a+ab=a+b | aa+b=ab |
| T6 | ab+ab=a | a+ba+b=a |
| T7 | ab+abc=ab+ac | a+ba+b+c=(a+b)(a+c) |
| T8 | a+b=ab | ab=a+b |
| T9 | ab+ac+bc=ab+ac | a+ba+cb+c=(a+b)(a+c) |
| T10(a) | fx1,x2,…,xn=x1⋅(f1,x2,…,xn+x1⋅f(0,x2,…,xn) |  |
| T10(b) | fx1,x2,…,xn=x1+f0,x2,…,xnx1+f1,x2,…,xn |  |

This page is intentionally left blank: you may use it as scratch “paper”.

Problem 1 (2.5 points)

Minimize the following function using K-maps into its minimal sum-of-products form.

fa,b,c,d=abcd+bcd+abc+abd+abcd

Problem 2 (2.5 points)

Make the following function using the minimum number of NOR gates.

fA,B,C,D=∏0,1,2,6,7,10⋅D3,8,9,14

Problem 3 (2.5 points)

Convert the following circuit into one using only NOR gates (and no inverters). Do not optimize the circuit except for the number of “inverters”.

F

B

C

A

Problem 4 (2.5 points)

Create a 2-to-3 active-low decoder with an active-low enable. Name your inputs {x2,x1, x0, E} (with x0 as the LSB).

Problem 5 (4 points)

Design a 4-to-16 active-low decoder using only 2-to-4 active-low decoder modules. No other gate types or modules may be used (you may use constant values). Assume that each 2-to-4 decoder has one active-low enable input, E. Name your 4-to16 decoder inputs {x3,x2,x1,x0} with x3 being the MSB.

Problem 6 (3 points)

Realize the expression Y=ABCD+ABCD+ABCD+ABCD+ABCD+ABCD+ABCD using the following using 2-to-4 active-low decoders (with active-low enables) and a single logic gate of any number of inputs.

Problem 7 (2.5 points)

Realize the expression Y=ABCD+ABCD+ABCD+ABCD+ABCD+ABCD+ABCD using a minimal number of 2-to-1 multiplexers.

Problem 8 (3 points)

Create a 5-bit subtractor using using NAND and XOR gates.

Problem 9 (5 points)

For the following circuit, complete the given timing diagram. “D2” is given for your convenience and will not be graded, but all other rows will be graded.

| X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLK |  |  |  |  |  |  |  |  |  |
| Q0 |  |  |  |  |  |  |  |  |  |
| Q1=D3 |  |  |  |  |  |  |  |  |  |
| D2 |  |  |  |  |  |  |  |  |  |
| Q2 |  |  |  |  |  |  |  |  |  |
| Q3 |  |  |  |  |  |  |  |  |  |
| Z |  |  |  |  |  |  |  |  |  |